THE UNIVERSITY OF TEXAS AT ARLINGTON

COMPUTER SCIENCE AND ENGINEERIG

LABORATORY 5 REPORT

ELECTRONICS LABORATORY

Submitted toward the partial completion of the requirements for CSE 3323-002

Submitted by,

Servando Olvera

1001909287

Date 10/26/2023

Lab 5: BJT Amplifier, NPN & PNP

Part 1: Amplifiers Designs

NPN Design:

PNP Design:

Part 2: LT Spice Simulations

NPN and PNP Circuit Diagram:

.OP Simulations:

Findings:

| NPN | PNP |
| --- | --- |
| Vc = 5.17 V Ic = 0.0094 A Ve = 0.94 V Vb = 1.65 Vb % error = 2.95% | Vc = 2.7 V Ic = 0.0092 A Ve = 7.01 V Vb = 6.36 V Vb % error = 0.95% |

8) What observation can you make about the values for R1, R2, RC, and RE for the PNP 	design relative to the values determined for the NPN design?

Re and Rc swap values and so do R1 and R2, for PNP relative to the NPN design.

9) If a -8V supply was available, could the NPN design be converted to the PNP design 	be changing ONLY the device and by replacing the 8V supply with a -8V supply?

No, this wouldn’t work since PNP and NPN behave differently regardless of a 8V or -8V 	supply

NPN -8V supply does not look similar in any way to value from PNP.

Part 3: AC LT Spice Analysis Simulations

NPN and PNP Circuit Diagram:

AC Simulations:

Findings:

1) Do a .AC simulation (decade, 10 pts per decade) from 1 Hz to 100 MHz. What is the midband gain in dB (at 10KHz)?

| NPN @10KHz -> gain = 9dB | PNP @10Hz -> gain = 9dB |
| --- | --- |

`

From this value, calculate numerical voltage gain and state the phase that goes with the gain.

| NPN G = 20 log (Vout/Vin) G/20 = log(Vout/Vin) 10^(G/20) = Vout/Vin Vout/Vin = 10^(5/20) Vout/Vin = 1.78 V/V Phase Shift = -120 | PNP G = 20 log (Vout/Vin) G/20 = log(Vout/Vin) 10^(G/20) = Vout/Vin Vout/Vin = 10^(5/20) Vout/Vin = 1.78 V/V Phase Shift = -120 |
| --- | --- |

What are the approximate –3 dB frequencies low and high. Include a screen shot in your report.

| NPN High -> 12.6 Hz Low -> 187.3 MHz | PNP High -> 12.8 Hz Low -> 169.2 MHz |
| --- | --- |

TRAN Simulation:

Do a .TRAN simulation with frequency = 1 KHz and amplitude = 1V.

Part 4: Building the Amplifiers

NPN Circuit Diagram:

Findings:

Measure Vc, Vb, Ve. Calculate Ic. Compare Ic and Vc to your goals.

Vc = 4.98 V

Vb = 1.66 V

Ve = 0.98 V

Ic = (8-4.98)/330 = 9.13 mA

Desired Vc = 5 V

Desired Ic = 10 mA

Measured values are close to desired values with some degree of error.

With function generator and scope, measure small signal midband gain / phase with output at collector, then f3dB low, and if possible f3dB high.

| Frequencyy (Hz) | Vin | Vout | Gain | Phase |
| --- | --- | --- | --- | --- |
| 1 | 1 | 0.3 | -10.4576 | 92 |
| 5 | 1 | 1.44 | 3.16725 | 96 |
| 10 | 1 | 1.9 | 5.575072 | 127 |
| 100 | 1 | 3.18 | 10.04854 | 176 |
| 500 | 1 | 3.22 | 10.15712 | 178 |
| 1k | 1 | 3.22 | 10.15712 | 179 |
| 5k | 1 | 3.22 | 10.15712 | 180 |
| 10k | 1 | 3.22 | 10.15712 | 180 |
| 50k | 1 | 3.38 | 10.57833 | 180 |
| 100k | 1 | 3.44 | 10.73117 | 180 |
| 1M | 1 | 3.38 | 10.57833 | 195 |
| 5M | 1 | 2.28 | 7.158697 | 238 |
| 10M | 1 | 1.42 | 3.045767 | 259 |

Mind-band Gain = 10 dB

F3dB low = 5 Hz

F3dB high = 10 MHz

What is the gain with output taken at the emitter?

Mind-band Gain (@10 kHz) = -0.291 dB

What is the max ppk output level at 1 KHz without significant distortion (on collector).

Max ppk @ 1 KHz  2.3 Vpp

PNP Circuit Diagram:

Findings:

Measure Vc, Vb, Ve. Calculate Ic. Compare Ic and Vc to your goals.

Vc = 3.05 V

Vb = 6.4 V

Ve = 7.12 V

Ic = (8-4.98)/330 = 9.13 mA

Desired Vc = 3 V

Desired Ic = 10 mA

Measured values are close to desired values with some degree of error.

With function generator and scope, measure small signal midband gain / phase with output at collector, then f3dB low, and if possible f3dB high.

| Frequency (Hz) | Vin | Vout | Gain | Phase |
| --- | --- | --- | --- | --- |
| 1 | 1 | 0.344 | -9.26883 | 107 |
| 5 | 1 | 1.08 | 0.668475 | 128 |
| 6 | 1 | 1.38 | 2.797582 | 120 |
| 10 | 1 | 1.72 | 4.710569 | 131 |
| 100 | 1 | 2.98 | 9.484325 | 172 |
| 500 | 1 | 3.02 | 9.600139 | 178 |
| 1k | 1 | 3.02 | 9.600139 | 179 |
| 5k | 1 | 3.04 | 9.657472 | 180 |
| 10k | 1 | 3.06 | 9.714429 | 180 |
| 50k | 1 | 3.22 | 10.15712 | 180 |
| 100k | 1 | 3 | 9.542425 | 171 |
| 1M | 1 | 3.12 | 9.883092 | 195 |
| 5M | 1 | 2.04 | 6.192603 | 243 |
| 10M | 1 | 1.27 | 2.076074 | 266 |

Mind-band gain = 10 dB

F3dB low = 6 Hz

F3dB high = 10 MHz

What is the gain with output taken at the emitter?

Mind-band Gain (@10 kHz) = -0.283 dB

What is the max ppk output level at 1 KHz without significant distortion (on collector).

Max ppk @ 1 KHz  1.9 Vpp

Why do we need C1/C3 between the function generator and the base bias resistors?

Their function might be to couple the signal to and from the transistor but block the DC current from flowing into it.